ATARI

CORPORATION

CONFIDENTIAL

TITLE

NEXT ASY.USED ON

APPLICATION

DRAWN BYDATE

CHECKEDDATE

ENGINEERDATE

APPROVALDATE

| 700026-XXX | JAGUAR |
| --- | --- |

| A. PAUL | 8/2/94 |
| --- | --- |

IC, CSTM,CPU,208P, QFP,JAG,MOTO

| REV. | DESCRIPTION | DATE | APPROVAL |
| --- | --- | --- | --- |
| A | PROD REL PER ECO 1711 |  |  |

NOTE: DEVICE TO BE MARKED AS SHOWN:

ã 1992  V 1.1

JAGUAR CPU

SC414200FT

FIGURE 1 PACKAGING DIMENSIONS

### TOM Pinout (Motorola version)

| 1 | VSS1 | 0V to output pads | Supply pin |
| --- | --- | --- | --- |
| 2 | VDD1 | 5V to output pads | Supply pin |
| 3 | XR0 | 4mA output | Video output |
| 4 | XR1 | 4mA output | Video output |
| 5 | XR2 | 4mA output | Video output |
| 6 | XR3 | 4mA output | Video output |
| 7 | XR4 | 4mA output | Video output |
| 8 | XR5 | 4mA output | Video output |
| 9 | XR6 | 4mA output | Video output |
| 10 | VDD1 | 5V to output pads | Supply pin |
| 11 | XR7 | 4mA output | Video output |
| 12 | XG0 | 4mA output | Video output |
| 13 | XG1 | 4mA output | Video output |
| 14 | XG2 | 4mA output | Video output |
| 15 | VSS2 | 0V to internal logic | Supply pin |
| 16 | XG3 | 4mA output | Video output |
| 17 | XG4 | 4mA output | Video output |
| 18 | XG5 | 4mA output | Video output |
| 19 | XG6 | 4mA output | Video output |
| 20 | XG7 | 4mA output | Video output |
| 21 | XB0 | 4mA output | Video output |
| 22 | XB1 | 4mA output | Video output |
| 23 | XB2 | 4mA output | Video output |
| 24 | XB3 | 4mA output | Video output |
| 25 | XB4 | 4mA output | Video output |
| 26 | XB5 | 4mA output | Video output |
| 27 | XB6 | 4mA output | Video output |
| 28 | XB7 | 4mA output | Video output |
| 29 | VSS1 | 0V to output pads | Supply pin |
| 30 | VDD1 | 5V to output pads | Supply pin |
| 31 | XHSL | 4mA output/TTL input | Video horizontal synchronization |
| 32 | XVSL | 4mA output/TTL input | Video vertical synchronization |
| 33 | XLP | CMOS input | Light-pen input |
| 34 | XINC | 4mA output | Video encrustation control |
| 35 | XEXPL | 4mA output | Expansion bus enable |
| 36 | XFC0 | 4mA output/TTL input | CPU function code |
| 37 | XFC1 | 4mA output/TTL input | CPU function code |
| 38 | VSS1 | 0V to output pads | Supply pin |
| 39 | XFC2 | 4mA output/TTL input | CPU function code |
| 40 | XDREQL | 4mA output/TTL input | CPU transfer request |
| 41 | XDTACKL | 4mA output | CPU transfer acknowledge |
| 42 | XRW | 4mA output/TTL input | Bus transfer direction |
| 43 | VDD1 | 5V to output pads | Supply pin |
| 44 | XSIZ0 | 4mA output/TTL input | Bus transfer size |
| 45 | XSIZ1 | 4mA output/TTL input | Bus transfer size |
| 46 | XINTL | 4mA output | CPU interrupt output |
| 47 | XDINT | CMOS input | DSP interrupt input |
| 48 | VSS3 | 0V to input pads | Supply pin |
| 49 | XBRL | 4mA output/TTL input | CPU bus request |
| 50 | XBGL | CMOS input | CPU bus grant |
| 51 | XBGA | 4mA output/TTL input | CPU bus grant acknowledge |
| 52 | XDSPCSL | 4mA output | DSP chip select |
| 53 | XRESETL | CMOS input | Master reset |
| 54 | VDD3 | 5V to input pads | Supply pin |
| 55 | XTEST | CMOS input | Test pin |
| 56 | XWAITL | CMOS input | Expansion bus wait request |
| 57 | XROMCSL1 | 4mA output | ROM chip select for cartridge |
| 58 | XROMCSL0 | 4mA output | ROM chip select for boot-strap |
| 59 | XDBGL | 4mA output | DSP bus grant |
| 60 | VSS3 | 0V to input pads | Supply pin |
| 61 | VDD3 | 5V to input pads | Supply pin |
| 62 | XDBRL1 | CMOS input | DSP bus request priority level 0 |
| 63 | XDBRL0 | CMOS input | DSP bus request priority level 1 |
| 64 | XPCLK | CMOS input | Internal processor clock |
| 65 | VSS2 | 0V to internal logic | Supply pin |
| 66 | XVCLK | CMOS input | Video clock |
| 67 | XMASKA0 | 4mA output | Address line for memory |
| 68 | XMASKA1 | 4mA output | Address line for memory |
| 69 | XMASKA2 | 4mA output | Address line for memory |
| 70 | XA0 | 4mA output/TTL input | System address bus |
| 71 | XA1 | 4mA output/TTL input | System address bus |
| 72 | XA2 | 4mA output/TTL input | System address bus |
| 73 | XA3 | 4mA output/TTL input | System address bus |
| 74 | XA4 | 4mA output/TTL input | System address bus |
| 75 | XA5 | 4mA output/TTL input | System address bus |
| 76 | XA6 | 4mA output/TTL input | System address bus |
| 77 | VSS3 | 0V to input pads | Supply pin |
| 78 | XA7 | 4mA output/TTL input | System address bus |
| 79 | XA8 | 4mA output/TTL input | System address bus |
| 80 | XA9 | 4mA output/TTL input | System address bus |
| 81 | XA10 | 4mA output/TTL input | System address bus |
| 82 | XA11 | 4mA output/TTL input | System address bus |
| 83 | XA12 | 4mA output/TTL input | System address bus |
| 84 | XA13 | 4mA output/TTL input | System address bus |
| 85 | XA14 | 4mA output/TTL input | System address bus |
| 86 | XA15 | 4mA output/TTL input | System address bus |
| 87 | XA16 | 4mA output/TTL input | System address bus |
| 88 | XA17 | 4mA output/TTL input | System address bus |
| 89 | XA18 | 4mA output/TTL input | System address bus |
| 90 | XA19 | 4mA output/TTL input | System address bus |
| 91 | XA20 | 4mA output/TTL input | System address bus |
| 92 | XA21 | 4mA output/TTL input | System address bus |
| 93 | VSS3 | 0V to input pads | Supply pin |
| 94 | XA22 | 4mA output/TTL input | System address bus |
| 95 | XA23 | 4mA output/TTL input | System address bus |
| 96 | VSS1 | 0V to output pads | Supply pin |
| 97 | VDD1 | 5V to output pads | Supply pin |
| 98 | XD24 | 4mA output/TTL input | System data bus |
| 99 | XD23 | 4mA output/TTL input | System data bus |
| 100 | XD8 | 8mA output/TTL input | System data bus |
| 101 | XD7 | 8mA output/TTL input | System data bus |
| 102 | XD25 | 4mA output/TTL input | System data bus |
| 103 | XD22 | 4mA output/TTL input | System data bus |
| 104 | VDD3 | 5V to input pads | Supply pin |
| 105 | XD9 | 8mA output/TTL input | System data bus |
| 106 | XD6 | 8mA output/TTL input | System data bus |
| 107 | XD26 | 4mA output/TTL input | System data bus |
| 108 | XD21 | 4mA output/TTL input | System data bus |
| 109 | XD10 | 8mA output/TTL input | System data bus |
| 110 | XD5 | 8mA output/TTL input | System data bus |
| 111 | XD27 | 4mA output/TTL input | System data bus |
| 112 | VSS3 | 0V to input pads | Supply pin |
| 113 | XD20 | 4mA output/TTL input | System data bus |
| 114 | VDD1 | 5V to output pads | Supply pin |
| 115 | XD11 | 8mA output/TTL input | System data bus |
| 116 | XD4 | 8mA output/TTL input | System data bus |
| 117 | XD28 | 4mA output/TTL input | System data bus |
| 118 | XD19 | 4mA output/TTL input | System data bus |
| 119 | VSS2 | 0V to internal logic | Supply pin |
| 120 | XD12 | 8mA output/TTL input | System data bus |
| 121 | XD3 | 8mA output/TTL input | System data bus |
| 122 | XD29 | 4mA output/TTL input | System data bus |
| 123 | XD18 | 4mA output/TTL input | System data bus |
| 124 | XD13 | 8mA output/TTL input | System data bus |
| 125 | XD2 | 8mA output/TTL input | System data bus |
| 126 | XD30 | 4mA output/TTL input | System data bus |
| 127 | XD17 | 4mA output/TTL input | System data bus |
| 128 | XD14 | 8mA output/TTL input | System data bus |
| 129 | VSS1 | 0V to output pads | Supply pin |
| 130 | XD1 | 8mA output/TTL input | System data bus |
| 131 | XD31 | 4mA output/TTL input | System data bus |
| 132 | VSS3 | 0V to input pads | Supply pin |
| 133 | VDD3 | 5V to input pads | Supply pin |
| 134 | XD16 | 4mA output/TTL input | System data bus |
| 135 | XD15 | 8mA output/TTL input | System data bus |
| 136 | XD0 | 8mA output/TTL input | System data bus |
| 137 | XMA10 | 16mA output/TTL input | DRAM multiplexed address bus |
| 138 | XMA9 | 16mA output/TTL input | DRAM multiplexed address bus |
| 139 | XMA8 | 16mA output/TTL input | DRAM multiplexed address bus |
| 140 | VSS1 | 0V to output pads | Supply pin |
| 141 | XMA7 | 16mA output/TTL input | DRAM multiplexed address bus |
| 142 | VSS1 | 0V to output pads | Supply pin |
| 143 | XMA6 | 16mA output/TTL input | DRAM multiplexed address bus |
| 144 | XMA5 | 16mA output/TTL input | DRAM multiplexed address bus |
| 145 | XMA4 | 16mA output/TTL input | DRAM multiplexed address bus |
| 146 | XMA3 | 16mA output/TTL input | DRAM multiplexed address bus |
| 147 | VDD1 | 5V to output pads | Supply pin |
| 148 | XMA2 | 16mA output/TTL input | DRAM multiplexed address bus |
| 149 | XMA1 | 16mA output/TTL input | DRAM multiplexed address bus |
| 150 | XMA0 | 16mA output/TTL input | DRAM multiplexed address bus |
| 151 | XD40 | 4mA output/TTL input | System data bus |
| 152 | VSS3 | 0V to input pads | Supply pin |
| 153 | XD39 | 4mA output/TTL input | System data bus |
| 154 | XD56 | 4mA output/TTL input | System data bus |
| 155 | XD55 | 4mA output/TTL input | System data bus |
| 156 | XD41 | 4mA output/TTL input | System data bus |
| 157 | XD38 | 4mA output/TTL input | System data bus |
| 158 | XD57 | 4mA output/TTL input | System data bus |
| 159 | XD54 | 4mA output/TTL input | System data bus |
| 160 | XD42 | 4mA output/TTL input | System data bus |
| 161 | VDD3 | 5V to input pads | Supply pin |
| 162 | XD37 | 4mA output/TTL input | System data bus |
| 163 | XD58 | 4mA output/TTL input | System data bus |
| 164 | VSS3 | 0V to input pads | Supply pin |
| 165 | VDD3 | 5V to input pads | Supply pin |
| 166 | XD53 | 4mA output/TTL input | System data bus |
| 167 | VSS1 | 0V to output pads | Supply pin |
| 168 | XD43 | 4mA output/TTL input | System data bus |
| 169 | XD36 | 4mA output/TTL input | System data bus |
| 170 | VSS2 | 0V to internal logic | Supply pin |
| 171 | XD59 | 4mA output/TTL input | System data bus |
| 172 | XD52 | 4mA output/TTL input | System data bus |
| 173 | XD44 | 4mA output/TTL input | System data bus |
| 174 | XD35 | 4mA output/TTL input | System data bus |
| 175 | XD60 | 4mA output/TTL input | System data bus |
| 176 | XD51 | 4mA output/TTL input | System data bus |
| 177 | XD45 | 4mA output/TTL input | System data bus |
| 178 | XD34 | 4mA output/TTL input | System data bus |
| 179 | XD61 | 4mA output/TTL input | System data bus |
| 180 | XD50 | 4mA output/TTL input | System data bus |
| 181 | XD46 | 4mA output/TTL input | System data bus |
| 182 | XD33 | 4mA output/TTL input | System data bus |
| 183 | XD62 | 4mA output/TTL input | System data bus |
| 184 | XD49 | 4mA output/TTL input | System data bus |
| 185 | XD47 | 4mA output/TTL input | System data bus |
| 186 | XD32 | 4mA output/TTL input | System data bus |
| 187 | XD63 | 4mA output/TTL input | System data bus |
| 188 | VSS1 | 0V to output pads | Supply pin |
| 189 | XD48 | 4mA output/TTL input | System data bus |
| 190 | XWEL0 | 16mA output | Memory write strobe |
| 191 | XWEL1 | 16mA output | Memory write strobe |
| 192 | XWEL2 | 4mA output | Memory write strobe |
| 193 | XWEL3 | 4mA output | Memory write strobe |
| 194 | XWEL4 | 4mA output | Memory write strobe |
| 195 | XWEL5 | 4mA output | Memory write strobe |
| 196 | XWEL6 | 4mA output | Memory write strobe |
| 197 | XWEL7 | 4mA output | Memory write strobe |
| 198 | XOEL0 | 16mA output | Memory output enable |
| 199 | XOEL1 | 8mA output | Memory output enable |
| 200 | VSS1 | 0V to output pads | Supply pin |
| 201 | VDD1 | 5V to output pads | Supply pin |
| 202 | XOEL2 | 8mA output | Memory output enable |
| 203 | XRASL0 | 16mA output | DRAM bank 0 row address strobe |
| 204 | XRASL1 | 16mA output | DRAM bank 1 row address strobe |
| 205 | XCASL0 | 16mA output | DRAM bank 0 column address strobe |
| 206 | XCASL1 | 16mA output | DRAM bank 1 column address strobe |
| 207 | VSS3 | 0V to input pads | Supply pin |
| 208 | VSS1 | 0V to output pads | Supply pin |

## TOM Pin Description

| XD[0..63] | The main data bus. Connects to DRAM, Jerry and 68000. Isolated from slower logic with TTL. TOM may simultaneously drive parts of the bus while inputting on others. This allows 16 and 32 bit processors to work with 64 bit DRAM. Narrower peripherals should be placed on the less significant end of the data bus. XD[0..15] are 8mA, XD[16..63] are 4mA outputs. |  |  |
| --- | --- | --- | --- |
| XA[0..23] | The main address bus. Connects to Jerry and the 68000. Isolated from slower logic with TTL. Narrow memory devices (less than 64bit) should not be connected to XA[0..2] but to XMASKA[0..2]. This allows TOM to break one wide request into several narrower cycles at different addresses. These are 4mA outputs. |  |  |
| XMA[0..10] | Multiplexed address bus. These signals carry the address to the DRAMs. The actual address signals to which each relates depends on the width of DRAM, the number of columns in the DRAM and whether outputting the row address or the column address. These are 16mA outputs. During reset these signals become inputs and 1K resistors tied either to ground or +5V are used to configure aspects of the system which cannot be set by software. They should be tied as follows. |  |  |
|  | XMA[0] | romhi | +5V |
|  | XMA[1] | romwidth[0] | 0V |
|  | XMA[2] | romwidth[1] | 0V |
|  | XMA[4] | nocpu | +5V |
|  | XMA[5] | cpu32 | 0V |
|  | XMA[6] | bigend | +5V |
|  | XMA[7] | extclk | 0V |
|  | XMA[8] | 68k | +5V |
| XMASKA[0..2] | Least significant address output. These are incremented when TOM breaks a wide cycle request into several narrow cycles at different addresses. These are 4mA outputs. |  |  |
| XROMCSL[0..1] | ROM chip selects. Active low 4mA outputs. |  |  |
| XRASL[0..1] | Row address strobes for each of two banks of DRAM. Once asserted (active low) each RAS remains asserted until an access from another row or a refresh cycle. These are 16mA outputs. |  |  |
| XCASL[0..1] | Column address strobes for each of two banks of DRAM. These are 16mA outputs. |  |  |
| XOEL[0..2] | Memory output enables. XOEL[0] applies to XD[0..15] and is a 16mA output, XOEL[1] applies to XD[16..32] and is an 8mA output, XOEL[2] applies to XD[32..63] and is an 8mA output. XOEL[0..1] should be used to control the direction of the data bus transceivers. |  |  |
| XWEL[0..7] | Memory write enables. XWEL[0] applies to XD[0..7], XWEL[1] applies to XD[8..15] and so on. These are 16mA outputs. |  |  |
| XPCLK | Processor clock input. This is the main clock used by the memory interface, object processor, graphics processor and blitter. The clock high time defines the CAS precharge time so the mark space should be controlled (most crystal oscillators are OK). |  |  |
| XVCLK | Video clock input. This clock is used by the video time-base and pixel logic. It should be identical to or somewhat slower than the processor clock XPCLK. |  |  |
|  | The video subsystem invokes the object processor by generating a pulse one video clock cycle wide. This is sampled by the processor clock. In order to guarantee that the pulse is seen the clocks should be identical or the video clock period should be greater by at least a few nanoseconds in order to satisfy sample and hold requirements and avoid problems relating to pulse thinning and clock jitter. |  |  |
| XRESETL | Active low reset input. Not a Schmitt input. |  |  |
| XWAITL | Active low wait input. Can be used to add wait states to memory and peripheral transfers. This input is tested on the rising clock edge prior to the last cycle in a transfer. DRAM transfers may not have wait states. |  |  |
| XDREQL | Active low transfer request. Used by external bus masters (68000 and Jerry) to request a memory cycle. This signal is connected to the 68000's address strobe. When internal bus masters own the bus. This signal is asserted during the first cycle of all transfers. This is a 4mA output. |  |  |
| XDTACKL | Active low transfer acknowledge. Used to signal to external bus masters that the cycle has completed. This signal is maintained until XDREQL is retracted. Read data is presented by TOM at the same time as XDTACKL. This is a 4mA output. |  |  |
| XRW | Read/write. This determines the direction of the current transfer. Driven by internal bus masters when they own the bus. This is a 4mA output. |  |  |
| XSIZ[0..1] | Transfer size. These determine the number of bytes to be transferred. They are connected to the 68000's LDS and UDS outputs so they also imply a[0] when the 68000 owns the bus. They are 4mA outputs. When Jerry or another external non 68000 microprocessor owns the bus they mean the following: |  |  |
|  | XSIZ[1] | XSIZ[0] | bytes |
|  | 0 | 0 | 4 |
|  | 0 | 1 | 1 |
|  | 1 | 0 | 2 |
|  | 1 | 1 | 3 |
|  | When an internal bus master owns the bus they become outputs and mean the following: |  |  |
|  | XSIZ[1] | XSIZ[0] | bytes |
|  | 0 | 0 | 8 |
|  | 0 | 1 | 1 |
|  | 1 | 0 | 2 |
|  | 1 | 1 | 4 |
| XDBRL[0..1] | Jerry bus request inputs. These two inputs request the bus for Jerry at one of two priorities. XDBRL[0] requests the bus at a priority just less than video. XDBRL[1] requests the bus at a priority greater than video but less than refresh. |  |  |
| XDBGL | Jerry bus grant output. Active low 4mA output. |  |  |
| XEXPL | Active low expansion bus enable. This 4mA output enables the data bus transceivers for all transfers to ROMs and peripherals. By dividing the data bus into a fast part and a slow part the parasitic capacitance can be reduced to keep speed high. This scheme also reduces the likelihood of static damage to the ASICs or DRAM. |  |  |
| XDSPCSL | Active low Jerry chip select. This 4mA output is asserted by TOM for all transfers in Jerry's 64k address range. |  |  |
| XINTL | Active low interrupt 4mA output. Used to interrupt the 68000. |  |  |
| XHSL, XVSL | Active low horizontal and vertical video syncs. May be programmed to output composite sync on XVSL. These are 4mA outputs. These may also be used as inputs so that external active low syncs can reset the internal vertical and horizontal time-bases in order to facilitate rapid genlocking. |  |  |
| XLP | Light pen input. |  |  |
| XR[0..7] XG[0..7] XB[0..7] | Red, green and blue outputs. These should be connected eight bit DACs to generate the analogue RGB required by monitors and video encoders. In practice an R-2R ladder can be directly attached to these outputs. These are 4mA outputs. |  |  |
| XINCL | Incrust output. This 4mA output may be used to switch between the internally generated video and an external video source on a pixel by pixel basis. The switch must be provided externally. |  |  |
| XDINT | Jerry interrupt input. Interrupts from Jerry are funnelled through this to the 68000. |  |  |
| XFC[0..2] | 68000 function code signals. If the microprocessor is a 68000 then these inputs are used to qualify transfer requests and decode interrupt acknowledge cycles. When an internal bus master owns the bus the value 101 is output on these 4mA outputs. |  |  |
| XBRL | 68000 bus request. This 4mA output is used to request the bus from the 68000. May also be used as an input for external bus masters. |  |  |
| XBGL | 68000 bus grant input. |  |  |
| XBA | 68000 bus grant acknowledge 4mA output. |  |  |
| XTEST | Test input. This is used for testing the chip in production. |  |  |

APPROVED SOURCE OF SUPPLY

| ATARI P/N 500436 | MFGR NAME MOTOROLA | MFGR P/N SC414200FT |
| --- | --- | --- |

APPROVED VENDORS LISTED ABOVE; NO SUBSTITUTIONS ALLOWED WITHOUT PRIOR APPROVAL FROM ATARI ENGINEERING