ATARI

CORPORATION

CONFIDENTIAL

TITLE

NEXT ASY.USED ON

APPLICATION

DRAWN BYDATE

CHECKEDDATE

ENGINEERDATE

APPROVALDATE

| 700037-01 | JAGUAR CD |
| --- | --- |

| A. PAUL | 8/15/94 |
| --- | --- |

IC, BUTCH ASIC PQFP

| REV. | DESCRIPTION | DATE | APPROVAL |
| --- | --- | --- | --- |
| A | PROD REL PER ECO 1728 |  |  |

500490   V2.0

Ó 1994  ATARI

Orbit 6299A

YYWW

| 1 PACKAGE NO. | LEAD COUNT | D | A | D | e |
| --- | --- | --- | --- | --- | --- |
| QFP100-D01 | 100 | 14.00 X 20.00 | 2.70 | 17.20 X 23.20 | 0.65 |

DC SPECIFICATIONS FOR INPUT/OUTPUT DEVICES

1.   OUTPUT PADS - 4.5V TO 5.5V, 0 TO 85C:

Iol @0.4v - 2mA;  Ioh@2.4v - 2mA <-2ma Pad (P_OBS2)

Iol @0.4v - 4mA;  Ioh@2.4v - 4mA <-4mAPad (P_OBS4)

Iol @0.4v - 6mA;  Ioh@2.4v - 6mA <-6mA Pad (P_OBS6)

Iol @0.4v - 8mA;  Ioh@2.4v - 8mA <-8ma Pad (P_OBS8)

Iol @0.4v - 24mA;  Ioh@2.4v - 24mA <-24ma Pad (P_OBS24)

2.  INPUT CELLS:

TTL - Vil - 0.8V; Vih - 2.0V

CMOS - Vil - 0.3 x VDD; Vih - 0.7 x VDD

TTL Schmitt - Min. Hystersis - 100mV

CMOS Schmitt - Min. Hystersis - 1V

3.  PULLUP/PULLDOWN DEVICES:

PULLUP

min pullup current - 15uA; MAX PULLUP CURRENT - 30uA (2.4 x 36)

PULLDOWN

MIN PULLDOWN CURRENT - 15uA; MAX PULLDOWN CURRENT - 30uA (2.4x36)

PIN LIST;

Pin     Name                     Spec

1       VSS                      SUPPLY

2       BOOTROM          OUTPUT - 6MA

3       EEPOMCS           OUTPUT - 6MA

4       A7                        INPUT - TTL

5       A13                      INPUT - TTL

6       A6                        INPUT - TTL

7       A14                      INPUT - TTL

8       A5                        INPUT - TTL

9       A15                      INPUT - TTL

10      A4                        INPUT - TTL

11      A16                      INPUT - TTL

12      A3                        INPUT - TTL

13      A17                      INPUT - TTL

14      A2                        INPUT - TTL

15      A18                      INPUT - TTL

16      ROM1                  INPUT - TTL

17      CARTROM          OUTPUT - 6MA

18      VSS                      SUPPLY

19      D15                      BIDIRECT - TTL, 6MA

20      D0                        BIDIRECT - TTL, 6MA

21      D7                        BIDIRECT - TTL, 6MA

22      D8                        BIDIRECT - TTL, 6MA

23      D14                      BIDIRECT - TTL, 6MA

24      D1                        BIDIRECT - TTL, 6MA

25      D6                        BIDIRECT - TTL, 6MA

26      D9                        BIDIRECT - TTL, 6MA

27      D13                      BIDIRECT - TTL, 6MA

28      D2                        BIDIRECT - TTL, 6MA

29      D5                        BIDIRECT - TTL, 6MA

30      VDD                     SUPPLY

31      VSS                      SUPPLY

32      D10                      BIDIRECT - TTL, 6MA

33      D12                      BIDIRECT - TTL, 6MA

34      D3                        BIDIRECT - TTL, 6MA

35      D4                        BIDIRECT - TTL, 6MA

36      D11                      BIDIRECT - TTL, 6MA

37      D31                      BIDIRECT - TTL, 6MA

38      D16                      BIDIRECT - TTL, 6MA

39      VDD                     SUPPLY

40      VSS                      SUPPLY

41      D23                      BIDIRECT - TTL, 6MA

42      D24                      BIDIRECT - TTL, 6MA

43      D30                      BIDIRECT - TTL, 6MA

44      D17                      BIDIRECT - TTL, 6MA

45      D22                      BIDIRECT - TTL, 6MA

46      D25                      BIDIRECT - TTL, 6MA

47      D29                      BIDIRECT - TTL, 6MA

48      D18                      BIDIRECT - TTL, 6MA

49      D21                      BIDIRECT - TTL, 6MA

50      VDD                     SUPPLY

PIN LIST CONTINUED;

51      VSS                      SUPPLY

52      D26                      BIDIRECT - TTL, 6MA

53      D28                      BIDIRECT - TTL, 6MA

54      D19                      BIDIRECT - TTL, 6MA

55      D20                      BIDIRECT - TTL, 6MA

56      D27                      BIDIRECT - TTL, 6MA

57      VSS                      SUPPLY

58      SYSCLK               INPUT - TTL

59      VSS                       SUPPLY

60      VDD                     SUPPLY

61      JRY_DATA          TRISTATE - 6MA

62      JRY_WCLK         TRISTATE - 6MA

63      JRY_CLK             TRISTATE - 6MA

64      CD_ERR              INPUT - TTL

65      CD_RST              OUTPUT -  6MA

66      CD_DATA           INPUT - TTL

67      CD_WCLK          INPUT - TTL

68      CD_CLK              INPUT - TTL

69      STROBE              BIDIRECT - TTL, 6MA

70      ACK                     BIDIRECT - TTL, 6MA

71      VDD                     SUPPLY

72      CART_IN             INPUT - TTL

73      RESETO              OUTPUT - 6MA

74      DATA                  BIDIRECT - TTL, 6MA

75      SUBCD                INPUT - TTL

76      INTR                   OUTPUT - 6MA/OD

77      OE0                      INPUT - TTL

78      OE1                      INPUT - TTL

79      WE2                     INPUT - TTL

80      VDD                     SUPPLY

81      VSS                      SUPPLY

82      WE0                     INPUT - TTL

83      RESET                 INPUT - TTL

84      A19                      INPUT - TTL

85      A20                      INPUT - TTL

86      A21                      INPUT - TTL

87      A22                      INPUT - TTL

88      TEST                   INPUT - TTL

89      DOOR                  INPUT - TTL

90      VDD                    SUPPLY

91      VSS                     SUPPLY

92      EEPROMDO       INPUT - TTL

93      EEPROMDI        OUTPUT - 6MA

94      EEPROMCK       OUTPUT - 6MA

95      A10                      INPUT - TTL

96      A9                        INPUT - TTL

97      A11                      INPUT - TTL

98      A8                        INPUT - TTL

99      A12                      INPUT - TTL

100     VDD                   SUPPLY

| LEAD COUNT |  | 100L |
| --- | --- | --- |
| BODY THICKNESS |  | 2.70 |
| FOOTPRINT (BODY +) |  | 3.20 |
| DIMENSIONS | TOLERANCE |  |
| A | MAX | 3.40 |
| A1 | 0.25 MIN | 0.25 MIN |
| A2 | +/- 0.10 | 2.70 |
| D |  | 17.20 |
| D1 | +/- 0.10 | 14.00 |
| E |  | 23.20 |
| E1 | +/- 0.10 | 20.00 |
| L | + 0.15/- 0.10 | 0.88 |
| e | basic | 0.65 |
| b | +0.05 | 0.30 |
| ccc | MAX | 0.10 |
| ddd |  | 0.12 NOM. |
| ? |  |  |

| LEAD COUNT |  | 100L |
| --- | --- | --- |
| BODY THICKNESS |  | 2.70 |
| FOOTPRINT (BODY +) |  | 3.20 |
| DIMENSIONS | TOLERANCE |  |
| A | MAX | 3.40 |
| A1 | 0.25 MIN | 0.25 MIN |
| A2 | +/- 0.10 | 2.70 |
| D |  | 17.20 |
| D1 | +/- 0.10 | 14.00 |
| E |  | 23.20 |
| E1 | +/- 0.10 | 20.00 |
| L | + 0.15/- 0.10 | 0.88 |
| e | basic | 0.65 |
| b | +0.05 | 0.30 |
| ccc | MAX | 0.10 |
| ddd |  | 0.12 NOM. |
| ? |  |  |

APPROVED SOURCE OF SUPPLY

| ATARI P/N 500490 | MFGR NAME | MFGR P/N |
| --- | --- | --- |

APPROVED VENDORS LISTED ABOVE; NO SUBSTITUTIONS ALLOWED WITHOUT PRIOR APPROVAL FROM ATARI ENGINEERING