QSoundÔ

for the Atari Jaguar

QSound Labs Inc.,

2748 - 37 Ave N.E.

Calgary, AB, Canada

T1Y 5L3

(403) 291-2492

fax: (403) 250 - 1521

QSound  technology is protected by patent and copyright laws..  It's use on the Atari Jaguar system is restricted to, and subject to, the licensing agreement signed with Atari.  All third parties interested in using QSound in Jaguar applications should check with Atari regarding this licensing agreement.

QSound is a patented, innovative process for generating a sound field that is unreferenced to the playback speakers.  It requires only traditional stereo playback equipment for reproduction, and provides enhanced audio imaging capabilities with startling contrasts.

Using the QSound process, sound sources can be placed in "virtual space": an arc approximately ±90 degrees in front of the listener, well outside the speakers.  The QSound pan positions which map this space are numbered 0 (far left) to 32 (far right).

µ §

For game developers, QSound provides a rich environment for audio interfacing.  For example, enemy fire can be heard in QSpace before the enemy appears on the screen; missiles launched off an F-16 can be heard to drop off the wing tip before they race off into the distance; explosions which are flown or driven past can appear to move beyond the player; background music can be given extra ambience and depth.

There are two ways of using QSound for Atari Jaguar games:

1. For sounds which can be preprocessed and require no dynamic control, the QSystem can be used.  This is a post production mixing system which results in stereo output.  For more information on the QSystem, contact QSound at the above address.

2. For sounds which are panned dynamically, Q1 has been implemented on the Jerry DSP on the Jaguar.  The rest of this document deals with Q1.

The implementation of Q1 on the Atari Jaguar system can be viewed as two black boxes; QSound and QSPan.

The QSPan module is called whenever a voice changes position within QSpace.  The QSound module is called every sample period in which at least one QSound voice is active.

The QSound module can process up to eight independently panned mono voices.  Regardless of the number of inputs, the output is always a stereo stream, which may be mixed with other stereo data before it is played back through the DAC.

The QSound process is dependent on the sampling rate.  The current implementation is for sampling rates near 22050 Hz, allowing for variances in Jaguar timing  (setting SCLK to either #18 or #19 will work).

µ §

The following addresses are required:

QSPan	-- start of the "qsound.o" module (dependent on where this is linked in)

QSound	-- QSPan + $36 (hex)

Descriptions of the two routines follow.  For further information or technical help, call Todd Simpson at the QSound address given above.

The file "qsound.o" is object code to be loaded into the Jerry DSP of the Jaguar.  It was two entry points which are documented below.  A full example showing how to call these functions is given in "sample.abs"

SUMMARY:

QSPan is called when a voice changes pan position.

QSound is called every sample period in which at least one QSound voice is active.

QSPan

input:

r20 = return address

r21 = voice to pan ( 0 through 7 )

r22 = new pan position ( 0 through 32 ($20))

output:

none

registers:

uses r20 through r24

notes:

takes about 25 instructions

example:

;;

movei	#QSPan,r5

movei	#Panned,r20		; return address for QSPan

movei	#0,r21			; pan voice 0

jump	T,(r5)

movei	#$20,r22		; pan position 32 - hard right

Panned:

;;

QSound

input

r16 = return address

r17 = number of QSound voices to process (1 to 8)

r18 -> input sample for each voice:

* consequtive 32 bit values

* bottom 16 bits represent input for each voice

output

r20 = left output (32 bits)

r22 = right output (32 bits)

registers:

uses r12 through r27

notes:

about 150 + 25*num_voices instructions.

example:

;;

...				; copy 16 bit inputs to #samples

movei	#After,r16		; return address for QSound

movei	#1,r17			; number of voices

movei	#QSound,r5

jump	T,(r5)			; call QSound

move	#samples,r18		; r18 -> input samples

After:

shrq	#16,r20			; outputs in 16 bits for DAC

shrq	#16,r22

...				; output to DAC

samples:		; up to 8 consequtive 32 bit locations

.dc.l 0		; voice 1 data

.dc.l 0		; voice 2 data

.dc.l 0		; voice 3 data

.dc.l 0		; voice 4 data

.dc.l 0		; voice 5 data

.dc.l 0		; voice 6 data

.dc.l 0		; voice 7 data

.dc.l 0		; voice 8 data

;;