J.A.Z.

Jaguar Audio System

Copyright 1995 Atari Corporation

- Confidential -

Beta Release, August 1995

# Introduction

JAZ is a new audio development system for the Atari Jaguar 64-Bit Interactive Mulitmedia System. It consists of a synthesizer with up to 14 voices, a sound editing tool and a variety of utility programs. It was designed for the usage for musicians and sound designers. A lot of effort was put into the user interface to hide as much technical details as possible. This documentation is intended for the programmers as well as  for musicians with technical background.

# Installation Overview

In order to work with JAZ you need the following hardware configuration:

- PC computer or Atari ST/TT/Falcon030 computer

- Jaguar Development System

- Atari Jaguar MIDI adaptor

- MIDI keyboard

- optional  Jaguar Mouse Interface with Atari ST compatible mouse

- if you are using a PC you will need a MIDI interface

This is the required software for JAZ:

- MS-DOS and optional Windows operating system or TOS

- MIDI sequencing software

- optional sample editing software

- Jaguar remote debugger (provided by Atari)

- JAZ Audio Tool (comes with JAZ)

- JAZ Synth Package (for the programmer - comes with JAZ)

- JAZ Synth Library  (for the programmer - comes with JAZ)

- JAZ Utility Package (comes with JAZ)

## Hardware Setup

Connect the Alpine Developemnt board on top of your Jaguar via a  parallel cable to your PC/TT. See your developer documentation about the proper set up. The parallel connection will be used to transfer the JAZ audio tool as well as data like audio samples.

Next you will have to connect your MIDI devices. The Jaguar MIDI interface has to be connected to the Alpine board:

Jaguar Alpine Board

MIDI

Out

In

Thru

MIDI Interface Connector (use flat cable for 12 pin connector)

Pin 1

Pin 1

Pin 1

Jumpers

Pin 1

Using a flat cable, the MIDI interface gets connected to the Jaguar. Please make sure that Pin 1 on the interface connects to pin 1 on the Alpine. Also, take a minute to set the jumper settings on the MIDI interface. The jumpers have to be set like this in order for the interface to work:

MIDI Interface Connector

Up (MIDI connectors)

J3

J2

J1

J4

Now that your Jaguar is equipped with MIDI, it’s time to connect your MIDI devices:

MIDI Keyboard

Parallel cable

Jaguar MIDI Interface

MIDI In

MIDI In

MIDI Out

MIDI Out

Alpine

PC/ST/TT

Jaguar

MIDI

Sequencer

Program

## Software Installation

If you have not installed your Jaguar development tools, please do so now. Refer to the Jaguar Developer Documentation on how to install the tools. To install JAZ, copy the.....

# Technical Overview

JAZ, the Jaguar Audio System, consists out of the follwoing parts:

JAZ Synth - Jaguar DSP Synthesize with up to 14 voices

- Modular Design, reprogrammable algorithms

- 7 oscillators:
	- Non-interpolated Sampler
	- Interpolated Sampler
	- Compressed Non-Interpolated Sampler
	- Compressed Interpolated Sampler
	- Fixed Wave Player
	- Polywave
	- FM

- Looping ADSR Envelopes

- LFOs

- Filter

- Run time voice alloaction with priority scheme

- Internal sampler buffer for up to 8 samples

- Support for one surround sound voice

- Controller changes for pitch bend, volume and pan

JAZ Audio Tool

- Runs on the Jaguar, leaving all processing power of host computer for sequencing software

- Real-time MIDI support, multi-temberal

- Eas to use graphical user interface with mouse support

- Allows reprogramming of synth algorithms via graphical user interface

- Build-in file selector box for easy acces s of files on host computer

- Editable patch map table

- Writes JAZ perfomance file which includes all information needed for generating music in a Jaguar game

JAZ Build

- Turns a JAZ performance file + MIDI file into a Jaguar executable program

- Future version will have a Windows user interface

- Hides makefiles, assembler, linker and parser from musician

JAZ Tools

- MIDI Parser, parses a MIDI file (type 0 and 1) and outputs a data files containing JAZ synth events.

- Sndcmpx, compresses a sample into the JAZ compressed sampling format (2-1).

- Stripaif, strips an AIFF header of an AIFF sample file

- Sample files showing how to initialize the synth and play music

JAZ SFX Programming Library

- Library for firing sound effects and for changing synth paramters at run-time

## Roadmap for the Musician

The best way to get familiar with the audio capabilities of the Jaguar is to play around with the JAZ Audio Tool. To load JAZ follow these steps:

- If you are using Windows, open an MS-DOS command line interpreter window

- Type CD \JAGUAR\JAZ

- To start, type JAZ
	- this will start the Atari debugger and will load JAZ into the Jaguar
	- JAZ then load its standard performance file including sample (this might take a while)
	- Now you are ready to use JAZ (see JAZ Audio Tool section in this manual)

- Setup your MIDI keyboard and play some notes - you should hear them coming out of the speakers hooked up to the Jaguar.

- Read the JAZ Audio Tool Manual

- Create you own sounds or use those provided by Atari

- Setup a perfomance file in JAZ with the patches (including samples) you want to use

- Compose your music with a sequencer program and play the composed song on the Jaguar

- Save the composed music on the PC/TT as a MIDI file

- Save the JAZ performance file

- Use JAZ Build to listen to your song on the Jaguar (the difference is that now your song is in the Jaguar and is not coming from real-time MIDI)

- Hand the MIDI file, the perfomance file and the samples to the game programmer