Notes regarding emulators.

Emulator	Format			Comment

--------	------			-------

ICE-5100	AOMF51			code and symbolic information

Intel standard		code

NEC-7809/7811	NEC-symb		symbols

Intel standard		code

Ashling		Ashling-<proc>		code and symbolic information

Nohau		AOMF51			code and symbolic information

Intel standard		code

Symbolic		code and symbols

Ashling			code and symbols

AMC		Extended-tekhex		code and symbols

ICE-196		AOMF96			code and symbolic information

Intel standard		code

Pentica		Pentica-<letter>	symbols

Intel standard		code

Motorola		code

Mice with USD	Intel standard		code

Tektronix		code

Motorola		code

AOMF51			code and symbolic information

Symbolic		code and symbols

Mice with MSD-3	Intel standard		code

Motorola		code

Tektronix		code

AOMF51			code and symbolic information

Extended-tekhex		code and symbols

HP		HP-code			code

HP-symb			symbols

ZAX		ZAX			symbols

Signum		AOMF51			code and symbolic information

otherwise use Signum-supplied

conversion utility

Motorola	Motorola		code

Huntsville	Ashling-<proc>		code and symbolic information

Metalink	Symbolic		code and symbols

Orion		Symbolic		code and symbols (choose [F7]

Manx/Aztec in Orion's Unilab Software

Ziltek		Use Ziltek-supplied converter utility

Sofia

Iwasaki proice	Mpds-code

Please let us know if you are using another format or another

emulator than those specified above.

The HP 64000 system

To run the program on the emulator do the following:

1. Link your program and if need be, make sure that there exists

room for the monitor.

2. If the emulator needs an monitor, link with the HP linker,

(on the host or the HP 64000) placing it at the correct address.

3. Make sure that the HP 64000 system is correctly configurated.

4. Download the files created by XLINK (the emulation files) to

the HP 64000 system.

5. Load the program and the monitor into the Measurement System.

Note: All symbols beginning with "?" are modified so they will

begin with "_" instead.