ATARI

CORPORATION

CONFIDENTIAL

TITLE

NEXT ASY.USED ON

APPLICATION

DRAWN BYDATE

CHECKEDDATE

ENGINEERDATE

APPROVALDATE

| 700082-XXX | JAGUAR |
| --- | --- |

| J. SMITH | 2/9/95 |
| --- | --- |

RES LADDER 10K/20K 10 PIN SIP

| REV. | DESCRIPTION | DATE | APPROVAL |
| --- | --- | --- | --- |

RESISTER LADDER  R/2R  8BIT

ELECTRICAL SPECIFICATIONS:

RESISTANCE VALUE	10K/20K

RESISTANCE TOLERANCE	± 2% @25°C

LADDER ACCURACY	± 1/2 LSB, 0°C to 70°C

TEMPERATURE COEFFICIENT	±100ppm/°C

OPERATING TEMPERATURE RANGE	0°C to 70°C

MAXIMUM OPERATING VOLTAGE	150V or ÖPR

INSULATION RESISTANCE	³10,000 Megohms

POWER DISIPATION

PER PACKAGE	0.8W @ 70°C

PER RESISTOR	0.05W @ 70°C

MECHANICAL SPECIFICATIONS:

LEAD MATERIAL	COPPER ALLOY

SUBSTRATE MATERIAL	ALUMINA

RESISTOR MATERIAL	CERMET

BODY MATERIAL	CONFORMAL EPOXY RESIN

PACKAGING SPECIFICATIONS:

PACKAGE STYLE	10 PIN SINGLE IN-LINE

LEAD SPACING	.100±.010 IN.

L max.	.996 IN.

H max.	.256 IN.

MANUFACTURER	MANUFACTURER PART NUMBER

BECKMAN INDUSTRIAL	H354BAI-1425DAD

MURATA ERIE	RCS10L103G